CONCEPTION OF DIAGNOSTIC SYSTEM OF AGRICULTURAL MACHINES

Jerzy Langman*

Summary

The paper presents conception of diagnostic system of agricultural machines adapted to work in precision farming. These machines have to meet higher requirements than machines used in traditional farming. Using artificial neural networks allows to build a diagnostic conclusion system of on-board computer of agricultural machine.

1. Introduction

Machines used in the precision farming (called applicators) are more complicated in constructional and operational meaning than machines currently used. Measurement, recognition and control systems installed at the agricultural machines make that increase a role and a range of electronics applied to the agricultural machines.

In the face of high degree complexity of agricultural machines there is, beside the mentioned systems, a need to equip the agricultural machines in diagnosis and monitoring systems with build-in selfdiagnostic functions of sensors and servomechanisms installed in the machines as well as systems protecting the machines from being damaged in cases when requirements determined by the producer or resulting from currently performed work are not fulfilled.

2. Diagnostic system of agricultural machines

Introducing the precision farming creates a necessity of using technically serviceable machines and continuous monitoring of work quality of the machines in order that every technological operation relating to cultivation were done according to a plan of work settled early. Simultaneously, the operator of the machine has to be informed about the work quality during the whole time in order that the operator can react in good time to each abnormality of the machine work. Therefore, continuous information transfer has to be guarantied between the agricultural machine, the operator and the stationary computer of the farm remotely controlling the operation of the cultivation technological process currently performed. In this way the closed information transfer system is created, which is shown in the Fig. 1.

For improving the information transfer between the individual modules of the system, the on-board computer of the agricultural machine should be provided with a special supervisory and a decision system fulfilling following functions:

- collecting and data processing received from measurement sensors;

- giving information to the operator related to the technical state of the agricultural machine and its work quality;

- controlling the servomechanisms;

- information interchange with the stationary computer.

Fig. 1	Schematic diagram of information flow between agricultural machine, operator and stationary farm computer

The information given by the measurement sensors and collected during the machine work are used multipleably in the shown system and serves:

- controlling the working units of the agricultural machine;

- giving information to the operator related to the work quality performed by the machine;

- preparing the operational use history of the agricultural machine to aim planing the necessary repair and service and collecting the material for the data base of the farm stationary computer;

- detecting break-down of main units of the agricultural machine.

To sum, the information transferred from the measurement sensors located at the agricultural machine could be divided into two main groups:

- diagnostic information;

- controlling information.

A schematic diagram of the record and transfer information of the agricultural machine being a diagnostic object (the author’s proposal) is shown in Fig. 2.

The recorded information are necessary in order that the used agricultural machines could be fallen under the system of quality management (TQM) [PN-ISO 9004-4+AC4]. One of the main conditions of practical application of the system of quality management at the agriculture is the precise documentation of running the cultivation process and continuous supervising of the quality work of the agricultural machines. As on the work quality directly influences the technical state of the agricultural machine, all main construction joints of the agricultural machines have to be monitored additionally. On the one hand falling under this system the agricultural machines raises new requirements to used agricultural machines but on the other hand bring decreasing in using cost and capital expenditure on production means as well as increasing a rate of production and improving the quality work of the machines.

In order that the information received from the sensors could be used in the effective way, the information should be submitted to primary treatment process in the real time and than stored or given to a module input of the diagnostic inference. As the processes have place in the main joints of the agricultural machines and are non-linear in more cases and their processing is difficulty in the traditional way, a classification method of measurement data and diagnostic interference using elements of artificial intelligence should be developed. This method should be a base for developing an operating system of the on-board computer of the agricultural machine  which can conduct the monitoring function of the main construction joints, the storing primary processed data received from the measurement sensors and the diagnostic inference (the diagnosis). Requirements made to the operational system, with which the on-board computer of the agricultural should be provided are large.

Fig. 2	Schematic diagram of information flow in agricultural machinery as diagnostic object: 1  sensors, 2-servomechanisms , 3  sensor data handling module, 4  servomechanisms control module, 5  data archiving module, 6  diagnostic reasoning module, 7  operator monitor, 8  output module to external diagnostic system module

This system should:

- be installed to the on-board computer of the agricultural machine;

- be adaptably;

- be tuneably;

- have an open architecture allowing correction;

- occupy a small part of operational memory of the on-board computer.

All these requirements can be fulfilled by developing the operational system using the artificial neural network. The author puts in the practice the simulation research on the use of the artificial neural network to solve three basic problems of the diagnostic inference, namely:

1.	the identification problem of the agricultural machine basing on the unserviceability symptoms;

2.	the problem of processing of analog diagnostic signals and their classification;

3.	the problem of processing of characteristics and their classification.

The carried out simulations aiming the prove of suitability of artificial neural networks for solving the diagnostic inference problems and for determining their best architecture allow to formulate following observations:

1.	All three basic problems of the diagnostic inference can be solved using two-layer artificial neural network of the back propagation error type;

2.	The functions of the activation each layer in all cases are: a hidden layer-a hyperbolic tangent function, an outputlayer - a sigmoidal function;

3.	All network had an inputlayer, one hidden layer and an output layer.

3. Conclusion

1.	The agricultural machines working in the precision farming system should have installed the on-board computers provided with the module of the diagnostic inference.

2.	The artificial neural networks allow developing the efficient and simple in exploitation modules of the diagnostic inference for all diagnostic cases.

* Dr. hab. Inž. Jerzy Langman 
Katedra Mechaniki Technicznej, Akademia Rolnicza w Krakowie
E-mail: rtlangma@cyf-kr.edu.pl